Post17.sys - PostScript device driver

=====================================

P. Jarvis.      (pj@doc.ic.ac.uk)

=================================

Introduction

------------

POST17.SYS is a small device driver which converts plain text to

PostScript. It was initially written to enable the NUPOP mail

program to output to a PostScript printer. The output is assumed

to be on A4 pages in portrait mode, and is printed using the

Courier font. There is an option to automatically wrap long lines

at column eighty.

The code is based on POSTPRN - a similar driver by Russ Nelson

(nelson@clutx.clarkson.edu).

This code (like POSTPRN) is freeware, no registration, no nagging,

no hassle, and no guarantee!

Installation

------------

The file POST17.SYS should be copied to the root directory on

the boot disk and the following line added to the CONFIG.SYS

file:

DEVICE=C:\POST17.SYS

(assuming the boot disk is drive C:)

Two parameters may be specified on the above line. These are:

/W    This causes the lines to be wrapped after column 80

/Pn   Output to LPTn: where n is either 1 or 2

Usage

-----

Once installed, any output sent to LPT3: will be converted to

PostScript and output via the selected parallel port (LPT1: by

default).

Comments

--------

The driver was written for a specificaly to allow the NUPOP mail

program to print directly to a PostScript printer. Normally NUPOP

can output only ASCII, and then only to parallel ports. Installing

the device driver worked except for when NUPOP checked the printer

status. NUPOP appeared to bypass the driver for a status check

thus trying to test for a none existant printer. To overcome this

the INT17 vector is now intercepted and any status requests for

LPT3 are re-directed to the selected output port.

The other explanation for the status not being processed properly

is that I have not written the driver correctly! Anyone with insight

on this is welcome to contact me at the above Email address.

Paul                25/02/1993