INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC 1/SC 29/WG 11 N3021

October 1999

Title: Study on Internet draft for the carriage of MPEG-4 on IP

Source: Systems

Status: Approved

The following text has been proposed as extension of the "RTP Payload Format for MPEG-4 Streams" draft-ietf-avt-rtp-mpeg4-01.txt.

It has been recognized that RTP payload formats for raw MPEG-4 elementary streams may be beneficial in some application scenarios that do not desire to use MPEG-4 Systems tools, such as scene description or object descriptors. The proposed specification should be analyzed and improved jointly between IETF and MPEG experts.

[Proposed revision on draft-ietf-avt-rtp-mpeg4-01.txt"RTP Payload Format for MPEG-4 Streams"]

(Add the following paragraph at the beginning of 3."Payload Format")

Applications that want to make use of the ISO/IEC 14496-1 capabilities [1], e.g. object descriptor and scene description, use the RTP payload format defined in 3.1.

(Move Figure 2 to section 3.1)

(Change the title of section 3.1 to from “RTP Header Fields Usage” to “RTP Header Fields Usage for SL packetized stream”)

(Add the following paragraph just before 3.1)

Applications that want to use no other ISO/IEC 14496 technology except ISO/IEC 14496-2 ("MPEG-4 Visual") or ISO/IEC 14496-3 ("MPEG-4 Audio") data streams use RTP payload format defined in 3.2 and 3.3. In this case, a fragment of a Compression Layer bitstream SHALL be directly mapped into a RTP packet. The RTP header fields usage and the fragmentation rule for the carriage of MPEG-4 Visual bitstreams and MPEG-4 Audio bitstreams are described in 3.2 and 3.3, respectively.

(Add the following sections just after 3.1)

3.2 RTP Packetization of MPEG-4 Visual bitstream

This section specifies the RTP payload format when MPEG-4 Visual content is directly carried over RTP without using MPEG-4 Systems capabilities.

When the short video header mode is used, RTP payload format for H.263 specified in the relevant RFCs or other standards MAY be used.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P|X|  CC   |M|     PT      |       sequence number         | RTP

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                           timestamp                           | Header

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|           synchronization source (SSRC) identifier            |

+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+

|            contributing source (CSRC) identifiers             |

|                             ....                              |

+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+

|                                                               | RTP

|       MPEG-4 Visual stream (byte aligned)                     | Payload

|                                                               |

|                               +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               :...OPTIONAL RTP padding        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure x - An RTP packet for MPEG-4 Visual stream

3.2.1 RTP header fields usage for MPEG-4 Visual

Payload Type (PT) field is used as described in 3.1.

Extension (X) bit: Defined by the RTP profile used.

Sequence Number: Increment by one for each RTP data packet sent. It starts with a random initial value for security reasons.

Marker (M) bit: The marker bit is set to one when the RTP packet contains the configuration information (see 6.2.1 "Start codes" of ISO/IEC 14496-2[2]), Group_of_VideoObjectPlane(), and/or the first fragment of an intra-coded VOP. Otherwise, set to zero.

This bit is used to indicate that the RTP packet is a point for random access, error-recovery using an intra-coded VOP, and recovering the configuration information.

Timestamp: The timestamp indicates the composition time, or the presentation time in a no-compositor decoder by adding a constant random offset for security reasons. For a video object plane, it is defined by vop_time_increment (in units of 1/vop_time_increment_resolution seconds) plus the cumulative number of whole seconds specified by module_time_base and time_code of Group_of_VideoObjectPlane() if present. In the case of interlaced video, a VOP consists of lines from two fields and the timestamp indicates the composition time of the first field. If the RTP packet contains only configuration information and/or Group_of_VideoObjectPlane(), the composition time of the subsequent VOP in the coding order is used.

Unless specified by an out-of-band means, the resolution of the timestamp is set to its default.

SSRC, CC and CSRC fields are used as described in RFC 1889 [5].

3.2.2 Fragmentation of MPEG-4 Visual bitstream

A fragmented MPEG-4 Visual bitstream is mapped directly onto the RTP payload without any addition of extra header fields or removal of any Visual syntax elements. The following rules apply for the fragmentation.

(1) A header of the configuration information function and Group_of_VideoObjectPlane() SHALL be placed at the beginning of the RTP payload (just after the RTP header) or just after the header of the syntactically upper layer function.

(2) If one or more headers exist in the RTP payload, the RTP payload SHALL begin with the header of the syntactically highest function.

(3) A header SHALL NOT be split into a plurality of RTP packets.

(4) Two or more VOPs SHALL be fragmented into different RTP packets so that one RTP packet consists of the data bytes associated with an unique presentation time (that indicated to the timestamp field in the RTP packet header).

(5) A single video packet SHOULD NOT be split into a plurality of RTP packets. The size of a video packet SHOULD be adjusted such that the resulting RTP packet is not larger than the path-MTU.

Here, header means the header of the configuration information functions (VisualObjectSequence(), VisualObject(), VisualObjectLayer()) and the entry point functions for a elementary stream(Group_of_VideoObjectPlane(), VideoObjectPlane(), video_plane_with_short_header(), MeshObject(), FaceObject()), the header of gob_layer(), and video_packet_header().

3.2.3 Examples of packetized MPEG-4 Visual bitstream

Considering that MPEG-4 Visual is used on a wide variety of networks from several Kbps to many Mbps, from guarantied networks with almost error-free to mobile networks with high error rate, it is not desired to apply too much restriction on the fragmentation like a single video packet shall always be mapped on a single RTP packet. On the other hand, a careless media unaware fragmentation will cause degradation of the error resiliency and the bandwidth efficiency. The fragmentation criteria described in 3.2.2 are flexible but to define the minimum rules to prevent the meaningless fragmentation of e.g. splitting a header into packets.

For video coding media such as H.261 or MPEG-1/2, the additional media specific RTP header works effectively for recovering e.g. a picture header corrupt by packet losses. However, there is an error resilience functionality inside MPEG-4 Visual to recover corrupt headers. This functionality can commonly be used on RTP/IP network as well as other networks. (H.223/mobile, MPEG-2/TS, etc.) Therefore, there is no strong reason to define MPEG-4 Visual specific extra RTP header fields.

Figure 3 shows examples of RTP packets generated based on the criteria described in 3.2.2

(a) is an example of the first RTP packet of an MPEG-4 visual object sequence. This packet contains the configuration information. According to the criterion (1), the header of VisualObjectSequence(VS header) is placed at the beginning of the RTP payload, and the headers of VisualObject and VisualObjectLayer(VO header, VOL header) follow it.

(b) is an example the RTP packet that contains Group_of_VideoObjectPlane(GOV). Following the criterion (1), the GOV is placed at the beginning of the RTP payload. It is a waste of RTP/UDP/IP header overhead to generate a RTP packet containing only a GOV whose length is 7 bytes. Therefore, (a part of) the following VOP can be placed in the same RTP packet as shown in (b).

(c) is an example that one video packet is packetized into one RTP packet. When the packet-loss rate of the underlying network is high, this kind of packetization is recommended. It is strongly recommended to set resync_marker_disable to 0 in the VOL header to enable adjustment of the video packet size. Even when the RTP packet containing the VOP header is discarded by a packet loss, the other RTP packets can be decoded by using the HEC(Header Extension Code) information in the video packet header. No extra RTP header field is necessary.

(d) is an example that more than one video packets are packetized into one RTP packet. This kind of packetization is effective to save the overhead of RTP/UDP/IP headers if the bit-rate of the underlying network is low. However, it will decrease the packet-loss resiliency because multiple video packets are discarded by a single RTP packet loss. The adequate number of video packets in a RTP packet and the RTP packet length depend the packet-loss rate and the bit-rate of the underlying network.

Figure 4 shows examples of RTP packets prohibited by the criteria of 3.2.2.

Fragmentation of a header into multiple RTP packets, like (a), will not only increase the overhead of RTP/UDP/IP headers but also decrease the error resiliency. Therefore, it is prohibited by the criterion (3).

When concatenating more than one video packets into a RTP packet, VOP header or video_packet_header() shall not be placed in the middle of the RTP payload. The packetization like (b) is not allowed by the criterion (2). This is because of the error resiliency. Comparing this example with Figure 3 (c), two video packets are mapped onto two RTP packets in both cases. However, there is a difference between the packet-loss resiliency. When the second RTP packet is lost, both video packets 1 and 2 are lost in the case of Figure 4 (b) whereas only video packet 2 is lost in the case of Figure 3 (c).

A RTP packet containing more than one VOPs, like (c), is not allowed.

+------+------+------+------+

(a) | RTP  |  VS  |  VO  | VOL  |

|header|header|header|header|

+------+------+------+------+

+------+-----+------------------+

(b) | RTP  | GOV |Video Object Plane|

|header|     |                  |

+------+-----+------------------+

+------+------+------------+  +------+------+------------+

(c) | RTP  | VOP  |Video Packet|  | RTP  |  VP  |Video Packet|

|header|header|    (1)     |  |header|header|    (2)     |

+------+------+------------+  +------+------+------------+

+------+------+------------+------+------------+------+------------+

(d) | RTP  |  VP  |Video Packet|  VP  |Video Packet|  VP  |Video Packet|

|header|header|     (1)    |header|    (2)     |header|    (3)     |

+------+------+------------+------+------------+------+------------+

Figure 3  Examples of RTP packetized MPEG-4 Visual bitstream

+------+-------------+  +------+------------+------------+

(a) | RTP  |First half of|  | RTP  |Last half of|Video Packet|

|header|  VP header  |  |header|  VP header |            |

+------+-------------+  +------+------------+------------+

+------+------+----------------+  +------+----------------+------+------------+

(b) | RTP  | VOP  | First half of  |  | RTP  |  Last half of  |  VP  |Video Packet|

|header|header|Video Packet (1)|  |header|Video Packet (1)|header|    (2)     |

+------+------+----------------+  +------+----------------+------+------------+

+------+------+------------------+------+------------------+

(c) | RTP  | VOP  |Video Object Plane| VOP  |Video Object Plane|

|header|header|        (1)       |header|       (2)        |

+------+------+------------------+------+------------------+

Figure 4  Examples of prohibited RTP packetization for MPEG-4 Visual bitstream

3.3 RTP Packetization of MPEG-4 Audio bitstream

In the case of the null SL packet header, specifications of the RPT packet SHOULD be used to transport MPEG-4 Audio bitstream in the LATM (Low-overhead MPEG-4 Audio Transport Multiplex) format[9].

3.3.1 RTP Packet Format

The LATM based MPEG-4 Audio bitstream format has an audio payload "audioMuxElement" between two syncwords. The audioMuxElement MUST be mapped directly onto the RTP payload without removal of any audioMuxElement syntax elements as shown in Figure 5.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P|X|  CC   |M|     PT      |       sequence number         |RTP

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                           timestamp                           |Header

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|           synchronization source (SSRC) identifier            |

+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+

|            contributing source (CSRC) identifiers             |

|                             ....                              |

+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+

|                                                               |RTP

:                 audioMuxElement (byte aligned)                :Payload

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 5 - An RTP packet for MPEG-4 Audio

It is required for the audioMuxElement to indicate the following information by an out-of-band means.

dataTypePresentEnable: If this information is ON, audioMuxElement SHALL include an indication bit "dataType". This bit indicates the data type of audioMuxElement is whether "setupData" element or "payloadMux" element.

If this information is OFF, audioMuxElement contains the payloadMux element.

errorProtectionEnable: This information indicates whether error protection is used.

Furthermore, the length of the audioMuxElement in bytes SHALL be required for the LATM-based audio decoder. This length MAY be equal with the length of the RTP payload. If the audioMuxElement is fragmented into several RTP packets, the cumulative length of all RTP packets MAY be used.

3.3.2 RTP Header Fields Usage

Payload Type (PT) field: This field is used as described in 3.1.

Marker (M) bit: The marker bit indicates audioMuxElement boundaries. This bit is set to one to mark the RTP packet contains an audioMuxElement or the last fragment of an audioMuxElement.

Timestamp: The timestamp indicates the composition time, or the presentation time in a no-compositor decoder. Timestamps are recommended to start at a random value for security reasons.

Sequence Number: Increment by one for each RTP packet sent. It starts with a random value for security reasons.

SSRC, CC and CSRC fields are used as described in RFC 1889 [5].

3.3.3 Fragmentation of MPEG-4 Audio bitstream

It is desireable to put one audioMuxElement per packet. The size of an audioMuxElement is tried to be adjusted such that the resulting RTP packet is not larger than the path-MTU. If this is not possible, the audioMuxElement MAY be fragmented accross several packets based on the following rules.

(1) "payloadMux" which consists of payload elements MAY be fragmented into several RTP packets so that one RTP packet consists of one or more payload elements. A payload element SHOULD NOT be fragmented.

(2) If the audioMuxElement consists of the configuration information "setupData", the configuration information SHALL NOT be fragmented.

3.4.  RTCP Packetization of MPEG-4 backward channel messages

This section specifies the usage of RTCP packets to carry the backword channel messages generated using the MPEG-4 Audio/Visual backword channel messaging functionalities, e.g. NEWPRED[10]. RTCP packets specified in this section SHALL ONLY be used when it is indicated by the profile and level indication the codecs have such functionalities. (e.g. Advanced Real Time Simple Visual Profile[10])

The MPEG-4 backward channel messages are transmitted on RTCP packets;

such as H.261 RTCP control packets [8].

The MPEG-4 backward message packets differ from normal RTCP packets in

that they are not transmitted to the normal RTCP destination transport

address for the RTP session (which is often a multicast address).

Instead, these backward packets are sent directly via unicast from the

decoder to the coder.  The destination port for these control packets is

the same port that the coder uses as a source port for transmitting RTP

(data) packets.  Therefore, these packets may be considered "reverse"

control packets.

As a consequence, these control packets may only be used when no RTP

mixers or translators intervene in the path from the coder to the

decoder.  If such intermediate systems do intervene, the address of

the coder would no longer be present as the network-level source

address in packets received by the decoder, and in fact, it might not

be possible for the decoder to send packets directly to the coder.

Some reliable multicast protocols use similar NACK control packets

transmitted over the normal multicast distribution channel, however

they typically use random delays to prevent a NACK implosion problem.

The goal of such protocols is to provide reliable multicast packet

delivery at the expense of delay, which is appropriate for

applications such as a shared whiteboard.

On the other hand, real-time video transmission is more sensitive to

delay and does not require full reliability.  For video applications

it is more effective to send the MPEG-4 backward message packets as

soon as possible, i.e. as soon as a loss is detected, without adding

any random delays.  In this case, multicasting the MPEG-4 backward

message packets would generate useless traffic between receivers since

only the coder will use them.  But this method is only effective when

the number of receivers is small.  The MPEG-4 backward message packets

are used only in point-to-point connections or in point-to-multipoint

connections when there are less than 10 participants in the conference.

3.4.1.  MPEG-4 Visual backward channel message packets definition

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P|   BMT   |  PT=RTCP_MP4B |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                              SSRC                             |

+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+

|    MPEG-4 Backward Channel Message Payload (byte aligned)     |

|                               +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               :             padding           |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

backward channel message type (BMT): 5 bits

Identifies the type of the MPEG-4 backward channel messages.

0: 	forbidden

1: 	MPEG-4 visual NEWPRED

2-63:	reserved

packet type (PT): 8 bits

The value of the packet type (PT) identifier is the constant

for MPEG-4 backward channel messages.

SSRC: 32 bits

SSRC is the synchronization source identifier for the sender of

this packet.

MPEG-4 Backward Channel Message Payload: variable

The syntax and semantics of the MPEG-4 backward channel messages

are defined in the ISO/IEC 14496.  In the case that the SL packet

header is used in ISO/IEC 14496-1[1], SL packet header is inserted

before the MPEG-4 backward channel message payload.

(Add the following reference at the bottom of 6."References")

[8] T. Turletti, C. Hitema, "RTP Payload Format for H.261 Video Streams",

RFC 2032, Octover 1996.

[9] ISO/IEC JTC1/SC29/WG11/N2794, "Technical Description of the Low Overhead MPEG-4 Audio Transport Multiplex (LATM)", July 1999.

[10] ISO/IEC 14496-2 FPDAM 1, July 1999.

END