TECHNICAL GUIDE to FX Uucico                                 version 1.0

FX UUCICO is Copyright (c) 1993, 1994 by Jorge Cwik. All rights reserved.

<jorge@satlink.net>

This documentation describes topics for advanced users and programmers

wanting to implement custom interfaces with FX Uucico. We are constantly

updating this notes acoording to quieres we receive. Check our server for

for the lastest update. ('help' to fx-dist@uufx.net)

I.     NETWORKING

II.    MULTITASKING

III.   16550A FIFO UARTS

IV.    TELEBIT modems

V.     EXIT LEVELS

VI.    STATUS files

VII.   SECURITY

VIII.  LIMITS AND MEMORY USAGE

IX.    CUSTOM INTERFACES WITH FX UUCICO

I.     NETWORKING

FX Uucico is fully networkable, you can run multiple uucicos concurrently

over a network, without any kind of conflict between them. FX will manage

all contention automatically.

From FX point of view, networking means 'sharing', running multiple Uucicos

in the same file system. Running it over a network doesn't neccessarily

implies sharing if used in single user mode. Multitasking two or more

Uucicos on the same machine has the same sharing effect as a network. You

usually need to load SHARE under DOS and peer to peer networks, but it is

normally not needed in Netware and other Client/Serves NOSes. It is always

implicitely loaded in WinNT and OS/2.

All configuration files are accessed in shared mode, which means that you

don't need to make them 'read only'. Writable files and jobs are accessed

in exclusive mode. Note that other Waffle's uucp utilities are not 'share'

aware, and can be networked in a limited fashion only. Configuration files

accessed by them should still be 'read only' (static, systems, paths, etc).

Receiving *.X files are written in a temporay file, they are renamed only

after they are complete. This will prevent UUXQT from trying to execute

commands before they are ready. The 'debug' and 'uucico' logs aren't any

more appended with the channel parameter. FX writes a different log per

session and later append it to the main file. Host are locked during the

connection, locked host are skipped by concurrent polls. An incoming call

to a locked system will be rejected as per uucp standard.

FX uucico doesn't need different channel parameters. It still uses it as

a flag to enable its sharing features, When 'channel' is not found in the

'static' file, FX will optimize performance for standard operation. At this

release, the optimization is done in two points: *.X files aren't saved with

a temporary name, and logs are written directly to the main file. Note that

the actual content of the 'channel' parameter is not used at all, it may be

blank. Other Waffle's program do need channels. The 'fx.share' configuration

option can be used to override sharing, independently of channel.

If you multitask more than one Uucico in the same machine, you must take

care that they do not try to access the same port at the same time. FX

doesn't currently lock ports.

II.    MULTITASKING

By default, the software releases its time-slice when idle. The method

used is compatible with DesqView, Windows, OS/2, WinNT, and others.

Releasing time-slice will help other multitasking software. It could be

very important when running multiple FX copies concurrently. Specially

if some drive a low speed and others a high speed connection.

It is normally not a good idea to let the operating system controlling

FX time-slices. Specifically we recommend to NOT enable the "detect idle

time" under Windows.

You may disable FX time-slice releasing with the option 'fx.multitask'.

Some multitaskers will automatically return control to a program receiving

a hardware interrupt. In our case, an incoming byte from the serial port

would wake-up FX task immediately. But some others fail to do this context

switch, in those cases you may get better results by disabling fx.multitask.

III.   16550A FIFO UARTS

The -z switch controls the internal driver buffering for the FIFO. The

syntax is -z<bytes>, where bytes is the trigger setting in the uart. To

disable the FIFO uart entirely use -z0. Using any other value, will bypass

FIFO autodetecion, FX will _force_ the internal driver to enter FIFO mode.

This may be helpful for not fully compatible UARTS and in some virtual

environments.

In some multitasking/networking environments, using the full FIFO buffers

at high DTE speed may produce data loss. If you experiment 'Rx' errors

with the FIFO in full mode, you may 'trigger' interrupts with FIFO buffers

partially full. Using -z1 (which is not the same as -z0) will give the

maximum reliabiity to the system.

Most setups don't need to mess with this parameter at all. It is used by

the internal driver only, and disregared in FOSSIL mode.

IV.    TELEBIT modems

TELEBIT are very powerful modems very popular in UUCP hosts. 'Wordlblazer',

the latest model at this moment, is an excellent choice. If both sides are

using FX UUCICO you would be foolish not to use the 'y' protocol, you would

get unbelievable throughput. We reached 2670 cps in compressed news between

two 'Wordblazers'.

Otherwise, you must use 64 bytes packet size to activate the modem 'spoofing'.

Without the spoofing these modems are very slow. The TELEBITs modem will not

spoof if _any_ of both sides requests packets size that are not 64 bytes.

Window size doesn't matter.

In long delay lines (like international calls), it may be better to do one of

these:

To transmit ascii non-compressed files, use protocol 'f'. If the other side

supports it, use 4Kb packet; the modems will not spoof, but you may get bet-

ter performance. If the quality of the line is good, try to connect in V32b

('Worldblazers' only).

NOTE:

The latest Telebit ROMs implement an "enhanced spoofing", which

spoofs large packets and windows 7. FX fully supports this new

enhanced spoofing in any packet size and windows combination.

V.     EXIT LEVELS

If 'fx.wQuirks' is on, FX Uucico returns 0 when one or more connections

were successfull, otherwise returns 1. Successfull means that the uucp

handshake has been acomplished. These values can be overriden with the

options "uu.exit and "uu.fail" respectively.

When 'fx.wQuirks' is off, FX returns 0 when the connection has been

completed, and one when it should be retried. Reasons for retries could

be failed scripts, loss of carrier, fatal timeouts, etc. If multiple

connections are requested with -s all or -s any, the exit level reflects

the status of the last connection attempted. Note that no connections due

to no queued jobs is interpreted as successfull.

FX may abort with higher exit levels, typically two or three, because of

configuration or I/O errors. Examples are, invalid parameters, some file

errors, or other unexpected situation.

If you need to know the precise reason and connection exit status, examine

the STATUS file documented below.

VI.    STATUS files

The format of these files have been completely changed. They are used for

two purposes, status and statistics. The 'uutraff' program, available

from our server, decodes these files and build several types of reports.

The format of these files, which is in binary, is available upon request.

You will trash the accumalated statistics if you mix incompatibles Uucicos,

but nothing else would be damaged though.

VII.   SECURITY

If you only poll other hosts and you don't receive incoming calls,

security is not a major issue. But when you operate Uucico as the

gate for an uucp server, and specialy when run unattended, you should

take care of possible security holes.

Uucp is not a fully secure protocol, yet with correct configuration

it would be enough to satisfy your normal security needs.

Two major options control which kind of access Uucico grants to remote

calls. The upload and download permits entries specifies which directories

are available besides the spool. Leaving both enties blank, will deny

any _remote_ request for uploading or downloading outside the spool.

The second issue is how to inforce that each host will 'take' its own

traffic and not other's one. Uucico by itself has now way to verify

if the remote host is the one it claims it is. This situation arises

because dial-in login is performed by other software, not by uucico.

If one valid uucp user enters the server with its own login name and

password, it may impersonate other system by using a different uucpname.

The solution has two steps. One is to link both pieces of software. The

front-end program must tell uucico the login name he verified with password.

The second step is to build a table that relate each login name to the

corresponding uucpname (or host).

For the first step, FX accepts the -u parameter that specifies the user

login name: "uucico -r0 -u <login_name>". If you are using waffle.exe

as your front-end program, use the %A variable.

The user name vs. uucpname table is done in the 'permits' file. For _each_

on of the host in 'systems' add an entry as the following one:

laser     /account=ulaser

Where 'laser' is the uucpname and 'ulaser' the login name. When FX

receives a call from a system that call itself 'laser', it will scan

for a 'permits' entry with the same name. Then, _if_ the -u parameter

has been passed on the command line _and_ there is an 'account' keyword

on the permit entry, FX will verify if they match. If they don't access

will not be granted and the connection will be rejected.

Notes:

Don't use the -p parameter. This will force FX to use a specific

'permits' entry and will disable the autolookup.

The login name and password in the 'systems' file are not used in

dial-in calls. They are the ones for accessing the remote host when

_you_ call him, and not viceversa.

There are other options in 'permits' that may be usefull for safety

and security measures.

VIII.  LIMITS AND MEMORY USAGE

There is no formal limit for the number of sytems, scripts, dialers

and permits. We have been running a host with over 200 uucp accounts.

The routines that read the configuration files have a finit limited

buffer. But using line continuation, there is no formal limit to the

total length.

FX doesn't requiere large amounts of memory. The exact number of bytes

depend on many parameters, and obviouslly also slightly change among

releases. The bare minimum is around the 80 kilobytes. But FX will

never use more than 150 kb. There is not much point to have more that

200 kb free before running this FX uucico release.

IX.    CUSTOM INTERFACES WITH FX UUCICO

We strongly recommend you use Uux for spooling jobs. FX Uux is rather small

and fast, the overhead is very little in comparison with the full compatibi-

lity you gain.

Uucico scans the spool for workfiles. Each known host has a personal

directory under <spool>, both outgoing and incoming jobs are placed

here. All outgoing jobs have a workfile with precise instructions for

Uucico. They are in ASCII text and have a conventional extension of 'CMD'.

Empty workfiles are allowed and they serve the purpose of forcing a poll

to a particular host. Each line in the workfile represent a single file

transaction, normally to send a data file to the remote host.

Most jobs normally have one or more data files associated. Lines on the

workfiles point to the neccessary data files. In many cases, one of the

data file is actually an execution file, to be intrepreted by the remote

host uucp (uuxqt program). FX Uucico supports 'E'xecution requests, they

avoid the transmission of the execution file. Incoming execution commands

are automatically converted to the standard format. Outgoing ones are

processed differently between hosts that support Exec request and those

who not. When the remote uucico doesn't support them, FX transforms the

request to traditional 'S' commands on the fly. Everything is handled

transparently and automatically, no external intervention is needed.

Workfiles with Execution requests have a different format, see the the FX

Uucp documentation.

Those that write programs that parse the logs created by Uucico, should

take special consideration to achieve maximum compatibility in future

releases. Try to allow as many changes as possible, many improvements in

the logging were not implemented because they breaked third party utilities.

For example if you parse a time field, let your code handle different

compatible formats like 20:13 and 20:13.45 . Do not assume a fixed amount

of spaces between tokens, perform string comparison without regarding case,

etc.

All files you don't modify should be accessed in read only mode with

deny none or deny write mode. Do not keep locks for extended periods of

time.

Incoming jobs are munged. Munging is the process of converting those Unix

filenames (D.laserC0123), to DOS conventions (0C01234.D). The munging

algorythm is documented with C source code in a small archive, fxmunge.zip,

available from our servers.